Road Roller YZ14H with Rexroth 

Structures of Road Roller YZ14H with Rexroth :
1. Road Roller YZ14H with Rexroth is equipped with reliable genuine Cummins engine
2. Single frequency & Double amplitude and super centrifugal force creates strong compacting effect
3. Rear-wheel hydraulic drive and stepless T/M.

Features of Road Roller YZ14H with Rexroth :
1. Reasonable configuration, reliable performance, and high working efficiency hydraulic components
2.Operator's station is designed for comfort and safety such as fully-adjustable, suspension-mounted operator's seat.

Applications of Road Roller YZ14H with Rexroth :
As a kind of construction machinery/construction equipment, this road roller YZ14H with Rexroth is widely used to compress the surface being rolled to ensure a smooth, even result on the construction site such as road-making, runway of the airport, and building.

Specifications of Road Roller YZ14H with Rexroth :
	Overall Dimension 

	1 
	Length 
	6000mm 

	2 
	Width(to outside of wheels) 
	2300mm 

	3 
	Height(To the top of the cab) 
	3055mm 

	4 
	Wheel base 
	2945mm 

	5 
	Tread 
	1560mm 

	6 
	Min. ground clearance 
	395mm 

	Main Technical Specification 

	1 
	Static line force 
	322N/cm 

	2 
	Dynamic line force 
	1370 N/cm 

	3 
	Operating weight 
	12000kg 

	4 
	Nominal Amplitude-high-low 
	1.8mm 0.9mm 

	5 
	Centrifugal-Force-high-low 
	285kN 142.5kN 

	6 
	Total stress force 
	36700(kg) 

	7 
	Static Weight at Drum 
	7000kg 

	8 
	Static Weight at Tires 
	7000kg 

	9 
	Frequency 
	31Hz 

	10 
	Min. turning radius(Outside of rear wheel) 
	5880mm 

	11 
	Drum diameter 
	1530mm 

	12 
	Drum width 
	2100mm 

	13 
	Drum thickness 
	36mm 

	14 
	Traveling speed(Km/h), 

	
	Stepless transmission 
	0~12.1 

	15 
	Gradient capacity 
	20% 

	Diesel engine 

	1 
	Model 
	Original Cummins engine 6BT5.9-C148 

	2 
	Type 
	Direct injection. Turbocharged. Water cooling 

	3 
	Rated output 
	110 kW 

	4 
	Number-Bore×Stroke 
	6-102×120 mm

	5 
	Total exhaust of cylinder 
	5.9 (L) 

	6 
	Rated speed 
	2200(r/min) 

	7 
	Max. Torque 
	590(N.M) 

	8 
	Starting type 
	Electric 

	9 
	Min. specific fuel consumption 
	229(g/Kw.h) 

	10 
	Net weight 
	410±20(kg)

	Driving System 

	1 
	Model 
	Static Hydraulic stepless driving 

	2 
	Type 
	Drive pump +drive motor + reducer 

	3 
	Model of drive pump 
	90R100

	4 
	Model of drive motor 
	90M075

	5 
	System pressure 
	42MPa

	6 
	Type of reducer 
	YZ14H.2A(Spur gear parallel shaft constant-mesh)

	7 
	Gear ratio of reducer 
	3.6

	Axle and Tire 

	1 
	Type of main reducer 
	spiral bevel gear, single stage 

	2 
	Gear ratio of main reducer 
	3.889 

	3 
	Type of final reducer 
	Single stage planetary 

	4 
	Gear of ratio of final reducer 
	4.941 

	5 
	Gear ratio 
	19.2155 

	6 
	Size of tire 
	23.1-26-12PR 

	Working Device Hydraulic System 

	1 
	Model of tandem pump(vibratory + steering) 
	HP051B498BIJM20-7BRAB06-1 

	2 
	Vibrating hydraulic system pressure 
	160(kgf/cm2) 

	3 
	Model of vibrating motor 
	HM051A878BEP020-7 

	4 
	Solenoid vibratory valve 
	ZF250B 

	Steering System 

	1 
	Type 
	Middle articulated frame. Full-hydraulic Steering 

	3 
	Model of redirector 
	BZZ1-E500C 

	4 
	System pressure 
	12MPa 

	5 
	Dimension of steering cylinder 
	Ф90*45*280(mm) 

	Brake System 

	1 
	Type of traveling brake 
	Single chamber brake valve Caplier disc brake Air over oil activate rear axle 

	2 
	Air pressure 
	6-8(kgf/cm2) 

	3 
	Type of parking brake 
	Hand brake 
Flexible shaft control Clamp brake 

	Oil Capacity 

	1 
	Fuel(diesel) 
	250 (L) 

	2 
	Engine lubricating oil 
	15 (L) 

	3 
	Oil for gear box 
	2.2(L) 

	4 
	Oil for hydraulic system 
	200 (L) 

	5 
	Driving axles(R) 
	Oil for main reducer 
	18 (L) 

	
	
	Oil for final reducer 
	2X3(L) 

	6 
	Oil for brake system 
	1.5( L) 

	7 
	Oil for vibrating drum 
	2X56(L) 


Changlin construction machinery/construction equipment: wheel loaders, road rollers, motor graders, backhoe loaders, pavers, Truck Mounted Crane , concrete truck mixers.
